Steps for Modifying OHCD Driver Code & Test the Zero Pkt Transfer

- Do Changes to the following files

\\WINCE500\public\common\oak\drivers\usb\hcd\ohcd2\Transfer.cpp

\\/WINCE500\public\common\DDK\INC\usbtypes.h

- Compile the OHCD2 Module alone to make sure there are no Compilation errors.

- Then Do a Build & Sysgen of the Platform.

- Install the New Driver from the Driver Disk provided. Older Drivers will not work for Zero Packet Transfer.

- Do a Ping with –l 150 bytes (which requires a Zero Packet Transfer).

- Also you can do a DiagnosticUtility Test with Packet Size in Multiples of 64 Bytes.

Note: Please make sure that while adding the Code Snippet, the Comments are proper so that it doesn’t give any compilation error.

Changes to OHCI Driver Code for Zero Packet Transfer

File: \\WINCE500\public\common\oak\drivers\usb\hcd\ohcd2\Transfer.cpp

The Code snippet in blue color in the following function has to be added to your code.

// ******************************************************************

// Scope: public

BOOL CTransfer::ScheduleTD()

//

// Purpose: Schedule TD for this transfer.

//

// return : TRUE: success schedule TD, FALSE: Error happens.

//

// Parameters: Nothing

//

// ******************************************************************

{

DWORD dwMaxTDDataSize = m_rPipe->GetMaxTDDataSize();

if (m_rPipe->IsTDQueueFull())

return FALSE;

if (m_dwNextToQueueIndex == (DWORD)-1) { // QUEUE Control header.

if (m_Transfer.lpvControlHeader !=NULL) {

PBYTE pSetupData = m_rPipe->GetTDQueueTailBufferVirtAddr();

DWORD dwOldPermissions =  SetProcPermissions(m_dwCurrentPermissions);

__try {

memcpy(pSetupData,m_Transfer.lpvControlHeader,sizeof(USB_DEVICE_REQUEST));

} __except( EXCEPTION_EXECUTE_HANDLER ) {

DEBUGMSG( ZONE_ERROR, (TEXT("CTransfer::ScheduleTD()(%s):Copy lpvControlHeader(%x) exception !!!\n"), m_rPipe->GetPipeType(),m_Transfer.lpvControlHeader ) );

}

SetProcPermissions(dwOldPermissions);

BOOL bReturn =m_rPipe->InitTDQueueTailTD(

this,

m_rPipe,

1, // InterruptOnComplete,

TD_SETUP_PID, // PID

TD::DATATOG_0, // Data Toggle (USB spec 8.5.2)

0,

sizeof( USB_DEVICE_REQUEST ), // MaxLength in (n-1) form

FALSE

);

ASSERT(bReturn==TRUE);

if (bReturn){

m_dwNextToQueueIndex=0;

m_rPipe->AdvanceTDQueueTail();

}

else

return FALSE;

}

else {

ASSERT(FALSE);

return FALSE;

}

}

//Newly Added Begin {

//To fix the Zero Packet Transfer Issue on OHCI Host Controllers

if( (m_dwNextToQueueIndex == 0) && (m_Transfer.dwBufferSize == 0) )

{

if((m_Transfer.dwFlags & USB_ZERO_PACKET_TRANSFER))

{

if(!m_rPipe->IsTDQueueFull() ) { // if this is TD not full yet.

BOOL bReturn =m_rPipe->InitTDQueueTailTD(

this,

m_rPipe,

1, // InterruptOnComplete,

((m_Transfer.dwFlags & USB_IN_TRANSFER)==0)?TD_OUT_PID:TD_IN_PID, // PID

TD::DATATOG_ED, // Data Toggle (USB spec 8.5.2)

0, //(m_Transfer.paClientBuffer == 0)?0:m_Transfer.paClientBuffer+m_dwNextToQueueIndex ,

0, // MaxLength in (n-1) form

TRUE //(m_dwNextToQueueIndex+dwLength>=m_Transfer.dwBufferSize && (m_Transfer.dwFlags & USB_SHORT_TRANSFER_OK)!=0)?TRUE:FALSE

);

ASSERT(bReturn==TRUE);

if (bReturn){

//m_dwNextToQueueIndex +=dwLength;

m_rPipe->AdvanceTDQueueTail();

}

else

return FALSE;

}

}

}

//}//End:Newly Added

while (!m_rPipe->IsTDQueueFull() && m_dwNextToQueueIndex < m_Transfer.dwBufferSize ) { // if this is TD not full yet.

DWORD dwLength = min(dwMaxTDDataSize,m_Transfer.dwBufferSize - m_dwNextToQueueIndex);

if ( m_Transfer.paClientBuffer == 0 && (m_Transfer.dwFlags & USB_IN_TRANSFER) ==0 ) { // We need Copy data

PBYTE pUserData = m_rPipe->GetTDQueueTailBufferVirtAddr();

DWORD dwOldPermissions =  SetProcPermissions(m_dwCurrentPermissions);

__try { // setting transfer status and executing callback function

memcpy( pUserData, (PBYTE)(m_Transfer.lpvClientBuffer) + m_dwNextToQueueIndex,dwLength);

} __except( EXCEPTION_EXECUTE_HANDLER ) {

DEBUGMSG( ZONE_ERROR, (TEXT("CTransfer::ScheduleTD()(%s)::CheckForDoneTransfers - exception setting transfer status to complete\n"), m_rPipe->GetPipeType() ) );

}

SetProcPermissions(dwOldPermissions);

}

BOOL bReturn =m_rPipe->InitTDQueueTailTD(

this,

m_rPipe,

1, // InterruptOnComplete,

((m_Transfer.dwFlags & USB_IN_TRANSFER)==0)?TD_OUT_PID:TD_IN_PID, // PID

TD::DATATOG_ED, // Data Toggle (USB spec 8.5.2)

(m_Transfer.paClientBuffer == 0)?0:m_Transfer.paClientBuffer+m_dwNextToQueueIndex ,

dwLength, // MaxLength in (n-1) form

(m_dwNextToQueueIndex+dwLength>=m_Transfer.dwBufferSize && (m_Transfer.dwFlags & USB_SHORT_TRANSFER_OK)!=0)?TRUE:FALSE

);

ASSERT(bReturn==TRUE);

if (bReturn){

m_dwNextToQueueIndex +=dwLength;

m_rPipe->AdvanceTDQueueTail();

}

else

return FALSE;

}

if (m_bNeedStatus &&  m_dwNextToQueueIndex >= m_Transfer.dwBufferSize && !m_rPipe->IsTDQueueFull()) {

// We need Status for Controller Endpoint.

BOOL bReturn =m_rPipe->InitTDQueueTailTD(

this,

m_rPipe,

1, // InterruptOnComplete,

((m_Transfer.dwFlags & USB_IN_TRANSFER)==0)?TD_IN_PID:TD_OUT_PID, // Reversed

TD::DATATOG_1, // Data Toggle (USB spec 8.5.2)

0 ,

0, // MaxLength in (n-1) form

FALSE

);

ASSERT(bReturn==TRUE);

if (bReturn){

m_rPipe->AdvanceTDQueueTail();

m_bNeedStatus = FALSE;

}

else

return FALSE;

}

return TRUE;

}

File:\\WINCE500\public\common\DDK\INC\usbtypes.h

//Add the #defined USB_ZERO_PACKET_TRANSFER at the End of the //Transfer Flags

// Flags for transfer functions

#define     USB_IN_TRANSFER				0x00000080

#define     USB_OUT_TRANSFER			0x00000000

#define     USB_NO_WAIT					0x00000100 //transfers //without events get completed immediately

#define     USB_SHORT_TRANSFER_OK		0x00000200 //allows the //transfer to be shorter than the buffer

#define     USB_START_ISOCH_ASAP			0x00000400

#define     USB_COMPRESS_ISOCH			0x00000800

#define     USB_SEND_TO_DEVICE			0x00001000

#define     USB_SEND_TO_INTERFACE		0x00002000

#define     USB_SEND_TO_ENDPOINT			0x00004000

#define     USB_DONT_BLOCK_FOR_MEM		0x00008000 // Don't block //waiting for memory allocations

//Newly Add this Begin{

#define     USB_ZERO_PACKET_TRANSFER			0x08000000

//}End